06-04-2018 Friday

MECE246 Midterm

Name:

Surname:

Number:

Q1) For the circuit below VCC=12 Volt, R1=106 Ohm, R2= 6×106 Ohm, RE=10600 Ohm, RC=2495×106 Ohm, VBE=0.7 Volt, IE=(β+1)IB where β=99. The transistor is working in active mode. Find VB, VE, VC, IB, IE, IC. (30 points)

Q2) In small signal AC analysis of a transistor acting as an amplifier the following procedures are carried out

- All DC sources are deactivated such as VCC (DC voltage sources are short-circuited and DC current sources are open-circuited)

- The capacitors are replaced by short circuit.

- The AC circuit model of the transistor (T-model preferably) replaces its DC model.

- Carry out the procedures explained above and obtain the AC circuit model for the amplifier given below: (10 points)

- Obtain the gain of the amplifier where Gain=voutvin (15 points)

- Obtain the input resistance of the amplifier (15 points)

The T-model of the transistor is given below (where K=β)

Q3) For the circuit below: D1 and D2 are ideal diodes V1=5 Volt, V2=7 Volt, R1=R2=1 kOhm, R=2 kOhm, Vin(t)= 20×Sin(2πft) Volt. Find Vout and draw it in the graph below: (30 points)