31-05-2019

MECE246 Final

Name:                                Surname:                                Number:

Q1) A FET amplifier structure in small signal AC analysis is given below. This circuit is examined for low frequency range and it should have 3 cut of frequencies due to the capacitors Cg, Cd and Cs. Find approximately the cut-off frequency due to capacitor Cg. In the calculations take r0=RL=RD=RS=1000 Ω, Rin=Rg=500 Ω, CS= 1 μFarad, gm=10-3 Siemens. (35 points)

Procedure to find the cut-off frequency due to Cg:

- Kill Vin (short circuit)

- Put a test source VTest to replace Cg

- Open circuit Cd and Cs

- Find the current originating from VTest (that current is ITest)

- RCg=VTest/ITest

- The cut-off frequency due to Cg is wcut-off_Cg=1/(CgRCg)

Q2) Two amplifier stages are connected as shown in the figure below: For this circuit I1=K1Ib,                              I2=K2VGS.

- Find the input resistance Rin=VinIin (7 points)

- Find the output resistance Rout. The procedure to find Rout is given below. (18 points)

- Kill Vin (short circuit)

- Put a test source VTest to  Vout

- Find the current originating from VTest (that current is ITest)

- Rout=VTest/ITest

Q3) For the circuit below MOSFET Q is used for biasing. In saturation mode the MOSFET has the following drain current equation.

ID=K1(VGS-Vt)2

The MOSFET gets into saturation mode if the following two conditions are satisfied at the operating point of the MOSFET:

VGS>Vt      and        VDS>VGS-Vt

The circuit and MOSFET parameters are given as RD=RS=1000 Ohm, VDD=10 Volt, R1=5×107 R2=5×107 Ohm, Vt=1 Volt, K1=0.5mAV2. Find VG, VD, VS and ID. (40 points)